/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Paper Counter - Giant UI Centered & Pure Linear Math & Undo Cal

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "usart.h"

#include "gpio.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

#include "fdc2214.h"

#include "OLED.h"

#include <stdio.h>

#include <math.h>

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

/* USER CODE BEGIN PV */

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

/* USER CODE BEGIN PFP */

#ifdef __GNUC__

#define PUTCHAR_PROTOTYPE int __io_putchar(int ch)

#else

#define PUTCHAR_PROTOTYPE int fputc(int ch, FILE *f)

#endif

PUTCHAR_PROTOTYPE

{

extern UART_HandleTypeDef huart1;

HAL_UART_Transmit(&huart1, (uint8_t *)&ch, 1, 0xFFFF);

return ch;

}

void Buzz_Beep(uint16_t ms);

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

// ==========================================

// 核心配置与算法变量

// ==========================================

#define CAL_POINTS 26

#define SHORT_CIRCUIT_RAW_THRESHOLD 80000000

const uint16_t Sheet_Nodes[CAL_POINTS] = {0, 1, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65, 70, 75, 80, 85, 90, 95, 100, 105, 110, 115, 120};

long long Cal_Data_Delta[CAL_POINTS] = {0};

uint32_t Air_Base_Raw = 0;

// 系统状态机

typedef enum {

STATE_CALIBRATING = 0,

STATE_READY,

STATE_MEASURED

} SystemState_t;

SystemState_t sys_state = STATE_CALIBRATING;

uint8_t cal_step = 0;

uint8_t valid_cal_points = CAL_POINTS;

// ==========================================

// 算法1：去极值平均滤波法

// ==========================================

uint32_t Get_Filtered_FDC_Data(void) {

uint32_t max_val = 0;

uint32_t min_val = 0xFFFFFFFF;

uint64_t sum = 0;

uint32_t temp = 0;

for(int i = 0; i < 22; i++) {

temp = FDC2214_ReadCH0_Raw();

if (temp > max_val) max_val = temp;

if (temp < min_val) min_val = temp;

sum += temp;

HAL_Delay(5);

}

sum = sum - max_val - min_val;

return (uint32_t)(sum / 20);

}

// ==========================================

// 算法2：分段线性插值与尾部外推 (两点纯线性占比)

// ==========================================

uint16_t Calculate_Sheets(long long current_delta) {

if (valid_cal_points < 2) return 0;

uint8_t is_ascending = (Cal_Data_Delta[valid_cal_points-1] > Cal_Data_Delta[1]);

if (is_ascending) {

if (current_delta <= Cal_Data_Delta[1]) return Sheet_Nodes[1];

// 区间内：两点之间纯线性插值

for (int i = 1; i < valid_cal_points - 1; i++) {

if (current_delta >= Cal_Data_Delta[i] && current_delta <= Cal_Data_Delta[i+1]) {

float frac = (float)(current_delta - Cal_Data_Delta[i]) / (float)(Cal_Data_Delta[i+1] - Cal_Data_Delta[i]);

return (uint16_t)round(Sheet_Nodes[i] + frac * (Sheet_Nodes[i+1] - Sheet_Nodes[i]));

}

}

// 区间外：最后两点纯线性外推

int last = valid_cal_points - 1, prev = valid_cal_points - 2;

float frac = (float)(current_delta - Cal_Data_Delta[prev]) / (float)(Cal_Data_Delta[last] - Cal_Data_Delta[prev]);

return (uint16_t)round(Sheet_Nodes[prev] + frac * (Sheet_Nodes[last] - Sheet_Nodes[prev]));

} else {

if (current_delta >= Cal_Data_Delta[1]) return Sheet_Nodes[1];

for (int i = 1; i < valid_cal_points - 1; i++) {

if (current_delta <= Cal_Data_Delta[i] && current_delta >= Cal_Data_Delta[i+1]) {

float frac = (float)(Cal_Data_Delta[i] - current_delta) / (float)(Cal_Data_Delta[i] - Cal_Data_Delta[i+1]);

return (uint16_t)round(Sheet_Nodes[i] + frac * (Sheet_Nodes[i+1] - Sheet_Nodes[i]));

}

}

int last = valid_cal_points - 1, prev = valid_cal_points - 2;

float frac = (float)(Cal_Data_Delta[prev] - current_delta) / (float)(Cal_Data_Delta[prev] - Cal_Data_Delta[last]);

return (uint16_t)round(Sheet_Nodes[prev] + frac * (Sheet_Nodes[last] - Sheet_Nodes[prev]));

}

}

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_USART1_UART_Init();

/* USER CODE BEGIN 2 */

printf("\r\n=== Paper Counter V10.4 (Pure Linear Math) ===\r\n");

OLED_Init();

OLED_Clear();

if (FDC2214_Init() == 0) {

OLED_ShowString(1, 1, "Sensor OK!    ");

Buzz_Beep(200);

} else {

OLED_ShowString(1, 1, "ERR: NO SENSOR");

while(1) { Buzz_Beep(300); HAL_Delay(300); }

}

HAL_Delay(500);

OLED_Clear();

char display_buf[20];

OLED_ShowString(1, 1, "Step 1: Cal    ");

sprintf(display_buf, "Insert: %-2d sht ", Sheet_Nodes[cal_step]);

OLED_ShowString(2, 1, display_buf);

OLED_ShowString(3, 1, "R:0            ");

OLED_ShowString(4, 1, "Press CAL KEY  ");

uint32_t current_raw = 0;

uint32_t last_ui_tick = 0;

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

// ==========================================

// 【后台任务】：实时刷新 Delta 数据 (仅未测量时)

// ==========================================

if (sys_state != STATE_MEASURED && (HAL_GetTick() - last_ui_tick > 150)) {

last_ui_tick = HAL_GetTick();

uint32_t live_raw = FDC2214_ReadCH0_Raw();

if (live_raw == 0) {

FDC2214_Init();

OLED_ShowString(3, 1, "D: SENSOR RST ");

} else {

char rt_buf[17];

if (cal_step == 0) {

sprintf(rt_buf, "R:%-12lu", live_raw);

} else {

long long live_delta = (long long)live_raw - (long long)Air_Base_Raw;

sprintf(rt_buf, "D:%-12lld", live_delta);

}

OLED_ShowString(3, 1, rt_buf);

}

}

// ==========================================

// 【按键 1】：CAL 键 (校准收集按键)

// ==========================================

if (HAL_GPIO_ReadPin(KEY_CAL_GPIO_Port, KEY_CAL_Pin) == GPIO_PIN_RESET)

{

HAL_Delay(20);

if (HAL_GPIO_ReadPin(KEY_CAL_GPIO_Port, KEY_CAL_Pin) == GPIO_PIN_RESET)

{

if (sys_state == STATE_CALIBRATING)

{

OLED_ShowString(4, 1, "Wait...       ");

HAL_Delay(800);

OLED_ShowString(4, 1, "Reading...    ");

current_raw = Get_Filtered_FDC_Data();

if (cal_step == 0) {

Air_Base_Raw = current_raw;

Cal_Data_Delta[0] = 0;

printf("[CAL 0 sht] Air Base: %lu, Delta: 0\r\n", Air_Base_Raw);

} else {

long long delta = (long long)current_raw - (long long)Air_Base_Raw;

Cal_Data_Delta[cal_step] = delta;

}

Buzz_Beep(100);

cal_step++;

if (cal_step < CAL_POINTS) {

sprintf(display_buf, "Insert: %-2d sht ", Sheet_Nodes[cal_step]);

OLED_ShowString(2, 1, display_buf);

OLED_ShowString(4, 1, "Press CAL KEY  ");

} else {

valid_cal_points = CAL_POINTS;

sys_state = STATE_READY;

OLED_Clear();

OLED_ShowString(1, 1, "Cal Finished! ");

OLED_ShowString(2, 1, "Ready to test.");

OLED_ShowString(4, 1, "Press MEASURE ");

}

}

while(HAL_GPIO_ReadPin(KEY_CAL_GPIO_Port, KEY_CAL_Pin) == GPIO_PIN_RESET);

}

}

// ==========================================

// 【按键 2】：MEASURE 键 (测试按键 / 撤销校准键)

// ==========================================

if (HAL_GPIO_ReadPin(KEY_MEASURE_GPIO_Port, KEY_MEASURE_Pin) == GPIO_PIN_RESET)

{

HAL_Delay(20);

if (HAL_GPIO_ReadPin(KEY_MEASURE_GPIO_Port, KEY_MEASURE_Pin) == GPIO_PIN_RESET)

{

// --- 撤销逻辑 ---

if (sys_state == STATE_CALIBRATING) {

if (cal_step > 0) {

cal_step--;

OLED_ShowString(4, 1, "Undo 1 Step... ");

Buzz_Beep(50); HAL_Delay(50); Buzz_Beep(50);

sprintf(display_buf, "Insert: %-2d sht ", Sheet_Nodes[cal_step]);

OLED_ShowString(2, 1, display_buf);

HAL_Delay(500);

OLED_ShowString(4, 1, "Press CAL KEY  ");

} else {

OLED_ShowString(4, 1, "Cannot Undo!   ");

Buzz_Beep(200);

HAL_Delay(500);

OLED_ShowString(4, 1, "Press CAL KEY  ");

}

}

// --- 测量逻辑 ---

else if (sys_state == STATE_READY || sys_state == STATE_MEASURED)

{

OLED_Clear();

OLED_ShowString(2, 1, "  Testing...  ");

OLED_ShowString(3, 1, " Please Wait  ");

HAL_Delay(800);

current_raw = Get_Filtered_FDC_Data();

if(current_raw > SHORT_CIRCUIT_RAW_THRESHOLD) {

OLED_Clear();

OLED_ShowString(1, 1, "!! WARNING !! ");

OLED_ShowString(2, 1, "SHORT CIRCUIT!");

OLED_ShowString(4, 1, "Press MEASURE ");

Buzz_Beep(1000);

sys_state = STATE_MEASURED;

} else {

long long current_delta = (long long)current_raw - (long long)Air_Base_Raw;

uint16_t result = Calculate_Sheets(current_delta);

OLED_Clear();

OLED_ShowBigNum(16, result, 3); // 巨型字体居中显示

Buzz_Beep(300);

sys_state = STATE_MEASURED;

}

}

while(HAL_GPIO_ReadPin(KEY_MEASURE_GPIO_Port, KEY_MEASURE_Pin) == GPIO_PIN_RESET);

}

}

// ==========================================

// 【按键 3】：ZERO 键 (跳过校准 / 强制重归校准)

// ==========================================

if (HAL_GPIO_ReadPin(KEY_ZERO_GPIO_Port, KEY_ZERO_Pin) == GPIO_PIN_RESET)

{

HAL_Delay(20);

if (HAL_GPIO_ReadPin(KEY_ZERO_GPIO_Port, KEY_ZERO_Pin) == GPIO_PIN_RESET)

{

if (sys_state == STATE_CALIBRATING)

{

if (cal_step >= 2) {

valid_cal_points = cal_step;

sys_state = STATE_READY;

OLED_Clear();

OLED_ShowString(1, 1, "Cal Skipped!  ");

sprintf(display_buf, "Saved %-2d Pts  ", valid_cal_points);

OLED_ShowString(2, 1, display_buf);

OLED_ShowString(4, 1, "Press MEASURE ");

Buzz_Beep(100);

} else {

OLED_ShowString(4, 1, "Need >=2 Pts! ");

Buzz_Beep(50); HAL_Delay(50); Buzz_Beep(50);

}

}

else if (sys_state == STATE_MEASURED || sys_state == STATE_READY)

{

sys_state = STATE_CALIBRATING;

cal_step = 0;

OLED_Clear();

OLED_ShowString(1, 1, "Step 1: Cal    ");

sprintf(display_buf, "Insert: %-2d sht ", Sheet_Nodes[cal_step]);

OLED_ShowString(2, 1, display_buf);

OLED_ShowString(3, 1, "R:0            ");

OLED_ShowString(4, 1, "Press CAL KEY  ");

Buzz_Beep(150);

}

while(HAL_GPIO_ReadPin(KEY_ZERO_GPIO_Port, KEY_ZERO_Pin) == GPIO_PIN_RESET);

}

}

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE;

RCC_OscInitStruct.HSEState = RCC_HSE_ON;

RCC_OscInitStruct.HSEPredivValue = RCC_HSE_PREDIV_DIV1;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE;

RCC_OscInitStruct.PLL.PLLMUL = RCC_PLL_MUL9;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV2;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_2) != HAL_OK)

{

Error_Handler();

}

}

/* USER CODE BEGIN 4 */

void Buzz_Beep(uint16_t ms) {

HAL_GPIO_WritePin(BUZZER_GPIO_Port, BUZZER_Pin, GPIO_PIN_SET);

HAL_Delay(ms);

HAL_GPIO_WritePin(BUZZER_GPIO_Port, BUZZER_Pin, GPIO_PIN_RESET);

}

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

* where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */